IoT-GPT Research Project

Student Contribution Guide

How to Learn, Build, and Contribute

Maintained by:  Gurdev Singh  |  gurdev.leo@gmail.com

Repository:  github.com/gsleo1976/iot-gpt-research

Version:  March 2026  —  Research Preview

Audience:  University students, hobbyists, beginner makers

1. What Is This Project?

IoT-GPT is a research project building an intelligent coordination layer for smart home devices. Instead of devices working in isolation, IoT-GPT gives them a shared brain — so a room heater knows the water heater is running, a smart bulb knows it is going to snow, and the whole home schedules itself around cheap electricity hours.

As a student contributor you will build real hardware, write real Python code, and run real experiments. By the end, you will have hands-on experience with IoT protocols, sensor programming, smart automation, and energy data analysis — skills that are directly relevant to careers in embedded systems, data engineering, and sustainable technology.

Core Idea: Today's "smart" devices are actually dumb loners. IoT-GPT makes them talk to each other — saving energy, reducing stress, and creating a calmer home.

2. Hardware Shopping List

You do not need all devices on day one. The list is ordered by priority — start with the first four items (under £50 / $60) and add more as you progress through the experiments.

2.1  Essential Starter Kit (~£50 / ~$60)

| # | Device | Purpose | Approx. Cost | Where to Buy |
| --- | --- | --- | --- | --- |
| 1 | Raspberry Pi 5 (4 GB) | Main computer — runs all automation logic | £55 / $75 | raspberrypi.com / Amazon India |
| 2 | Raspberry Pi USB-C Power Supply | Powers the Pi reliably at 5V 5A | £12 / $15 | raspberrypi.com / Amazon India |
| 3 | MicroSD Card 32 GB+ (Class 10) | Operating system & data storage | £8 / $10 | Amazon India (amazon.in) |
| 4 | DHT22 Temperature/Humidity Sensor | Measures indoor temperature & humidity | £5 / $6 | Amazon India (amazon.in) |
| 5 | Breadboard + Jumper Wires Kit | Prototype circuits without soldering | £5 / $6 | Amazon India (amazon.in) |
| 6 | HDMI cable + monitor (temp) | First-time Pi setup only | You may already have one | — |

2.2  Smart Lighting & ZigBee (~£40 / ~$50)

| # | Device | Purpose | Approx. Cost | Where to Buy |
| --- | --- | --- | --- | --- |
| 7 | SONOFF Zigbee 3.0 USB Dongle Plus | ZigBee coordinator — connects smart bulbs to Pi | £15 / $18 | Amazon India / sonoff.tech |
| 8 | IKEA TRADFRI E27 White Spectrum Bulb | Calm ambient signals — warm to cool white | £8 / $10 | IKEA India / Amazon India |
| 9 | IKEA TRADFRI E27 Colour Bulb | Full RGB colour signals (snow = blue, etc.) | £15 / $18 | IKEA India / Amazon India |
| 10 | Lamp or desk lamp with E27 socket | Holds the smart bulb for testing | £8 / $10 | Amazon India / IKEA India |

2.3  Energy Monitoring (~£30 / ~$36)

| # | Device | Purpose | Approx. Cost | Where to Buy |
| --- | --- | --- | --- | --- |
| 11 | TP-Link Kasa Smart Plug (with energy monitoring) | Measures real-time power draw of any device plugged in | £15 / $18 | Amazon India (amazon.in) |
| 12 | Second Smart Plug (any brand with energy metering) | Monitor a second device (e.g. kettle) simultaneously | £12 / $15 | Amazon India (amazon.in) |

Total Budget: Starter kit only: ~£50 / ~$60. Full setup (all items): ~£148 / ~$188. You can start all experiments with just the starter kit + ZigBee items.

3. Daily Learning Plan  —  12-Week Roadmap

This plan assumes 1–2 hours per day on weekdays and 2–3 hours on weekends. Each week ends with a hands-on mini-project so you always finish the week with something working.

PHASE 1  (Weeks 1–3)  —  Foundations

Week 1  —  Python & Raspberry Pi Basics

| Day | Topic | Task / Resource |
| --- | --- | --- |
| Mon | Python refresher — variables, loops, functions | Complete Python.org tutorial sections 1–4 ¦ python.org/about/gettingstarted |
| Tue | Working with files and JSON in Python | Read/write a JSON file; practice dict operations |
| Wed | Flash Raspberry Pi OS onto microSD | Download Raspberry Pi Imager; flash OS; first boot |
| Thu | SSH into Pi, install packages (pip, git) | Enable SSH in raspi-config; connect from laptop |
| Fri | Run your first Python script on the Pi | Write a hello.py; schedule it with cron |
| Sat | Requests library — calling a web API | Fetch weather from wttr.in using requests library |
| Sun | Mini-project: Weather forecast terminal app | Print 3-day forecast for your city with colour output |

Week 2  —  Sensors & GPIO

| Day | Topic | Task / Resource |
| --- | --- | --- |
| Mon | GPIO basics — digital in/out | Blink an LED using RPi.GPIO library |
| Tue | DHT22 wiring — 3-wire connection to Pi | Wire DHT22 to pins 3.3V, GND, GPIO4; read values |
| Wed | Reading DHT22 with Python | pip install adafruit-circuitpython-dht; read every 10s |
| Thu | Logging sensor data to CSV | Write timestamp, temp, humidity to data.csv every 60s |
| Fri | Matplotlib — plot your sensor data | Load CSV, plot 24-hour temperature chart |
| Sat | Threshold alerts — print when too hot/cold | Add logic: if temp > 24°C print "Too warm" |
| Sun | Mini-project: Indoor climate monitor | Run for 24 hours; plot results; identify patterns |

Week 3  —  MQTT Messaging Protocol

| Day | Topic | Task / Resource |
| --- | --- | --- |
| Mon | What is MQTT? Topics, subscribe, publish | Read: mosquitto.org/man/mqtt-7.html |
| Tue | Install Mosquitto broker on Pi | sudo apt install mosquitto mosquitto-clients |
| Wed | Publish a message from command line | mosquitto_pub -t home/temp -m "22.5" |
| Thu | Subscribe with Python (paho-mqtt) | pip install paho-mqtt; write subscriber.py |
| Fri | Publish sensor data to MQTT topic | Combine DHT22 reader + paho publish — pipe live data |
| Sat | Multiple topics — simulating two devices | Publish temp + humidity to separate topics |
| Sun | Mini-project: Live sensor dashboard in terminal | Subscribe to all topics; print latest values on screen |

PHASE 2  (Weeks 4–6)  —  IoT & Smart Devices

Week 4  —  ZigBee & Smart Bulbs

| Day | Topic | Task / Resource |
| --- | --- | --- |
| Mon | What is ZigBee? How ZigBee mesh works | Read: zigbee.org/learn + zigbee2mqtt.io/guide/getting-started |
| Tue | Install Zigbee2MQTT on Pi | Follow zigbee2mqtt.io installation guide |
| Wed | Pair your first IKEA TRADFRI bulb | Enable pairing mode in Zigbee2MQTT; factory reset bulb; pair |
| Thu | Control bulb brightness via MQTT | Publish to zigbee2mqtt/bulb/set — change brightness 0-254 |
| Fri | Control bulb colour temperature | Set color_temp (150=cool, 500=warm) |
| Sat | Read bulb state back via MQTT | Subscribe to zigbee2mqtt/bulb — read JSON state |
| Sun | Mini-project: Bulb as a calm notification | If DHT22 temp > 24°C → bulb turns warm amber |

Week 5  —  APIs & External Data Sources

| Day | Topic | Task / Resource |
| --- | --- | --- |
| Mon | REST APIs — GET requests and JSON parsing | Use wttr.in/London?format=j1 — parse JSON response |
| Tue | Open-Meteo weather API (no API key needed) | open-meteo.com — fetch 7-day forecast for your city |
| Wed | Sunrise/sunset API | api.sunrise-sunset.org — get today's sunrise time |
| Thu | Open-Meteo AQI — air quality index | Fetch hourly AQI; interpret pm2_5 values |
| Fri | Combining two APIs in one script | Fetch weather + AQI together; decide if window should open |
| Sat | Scheduling — run code at specific times | Use schedule library: pip install schedule |
| Sun | Mini-project: Sun-aware daily automation | Lights dim at sunset; morning weather check at 7am |

Week 6  —  Data Storage & Visualisation

| Day | Topic | Task / Resource |
| --- | --- | --- |
| Mon | SQLite basics — create, insert, query | import sqlite3; create sensor_data.db; store readings |
| Tue | Install InfluxDB on Pi | Follow influxdata.com/downloads — OSS version |
| Wed | Write sensor data to InfluxDB | pip install influxdb-client; write temperature points |
| Thu | Install Grafana on Pi | apt install grafana; connect to InfluxDB data source |
| Fri | Build a Grafana dashboard | Create panels: temperature, humidity, 24h trend |
| Sat | Set up Grafana alerts | Alert if temperature stays > 25°C for 30 minutes |
| Sun | Mini-project: 48-hour sensor recording | Log all sensor data for 2 days; analyse in Grafana |

PHASE 3  (Weeks 7–9)  —  IoT-GPT System Building

Week 7  —  Device Coordination Logic

| Day | Topic | Task / Resource |
| --- | --- | --- |
| Mon | JSON data structures for device attributes | Design a dict: {"name":"water_heater","watts":3500,"warmup_min":20} |
| Tue | Building the Device Attribute Registry (DAR) | Create devices.json with 5 devices; write a loader class |
| Wed | Conflict detection — which devices clash? | Write detect_conflicts(): if sum of simultaneous watts > 4000 |
| Thu | Simple scheduler — earliest start time | Given constraints, find safe start slots for each device |
| Fri | Off-peak electricity windows | Add peak_hours=[6,9,17,20] to DAR; find cheapest slots |
| Sat | OR-Tools introduction (optional advanced) | pip install ortools; model the scheduling as a constraint problem |
| Sun | Mini-project: Two-device scheduler | Schedule water heater + room heater with no overlap |

Week 8  —  Putting It All Together

| Day | Topic | Task / Resource |
| --- | --- | --- |
| Mon | Multi-sensor MQTT network | DHT22 on Pi + simulated second sensor; both publish |
| Tue | Coordinator script — subscribe to all topics | iot_gpt_brain.py: reads all sensors, makes decisions |
| Wed | Decision engine — if/then rules | If morning + cold → schedule water heat for 06:40 |
| Thu | Light as output — map decisions to bulb colour | Cold morning → warm white; snow tonight → blue glow |
| Fri | Energy budget tracking | Track daily kWh; if over budget → amber warning light |
| Sat | Testing and debugging | Simulate edge cases: sensor failure, API timeout, bulb offline |
| Sun | Mini-project: Full mini IoT-GPT demo | All sensors → brain → decisions → calm light signals working |

Weeks 9–12  —  Contribute to the Research

By week 9 you have a working prototype. The remaining weeks are about contributing to the actual research:

Run the provided experiments (Section 5) and log your results to a shared CSV format

Open a GitHub issue or pull request with your findings, improvements, or new sample code

Write up a short experiment report (1–2 pages) using the provided template

Optionally: implement a new use case and add it to the Device Attribute Registry

4. Your First Project  —  Weather-to-Light Signal

This is the very first thing you will build. It takes about 3–4 hours end-to-end and demonstrates the single most important idea in the project: technology informing without demanding. A smart bulb glows a calm colour based on tomorrow's weather — no phone, no notification, no interruption.

4.1  What You Will Build

End Result: A Python script on your Raspberry Pi that checks the weather forecast every hour. If snow or ice is forecast, the shoe-rack bulb glows blue. If it is a warm sunny day, it glows soft warm white. If rain is coming, it glows cool white. Running 24/7, silently.

4.2  Hardware Required

Raspberry Pi 5 (set up with Raspberry Pi OS)

SONOFF ZigBee USB Dongle (plugged into Pi)

IKEA TRADFRI E27 Colour Bulb (paired via Zigbee2MQTT)

Desk lamp or any E27 lamp socket

Internet connection on the Pi

4.3  Software Setup

Step 1 — Install Zigbee2MQTT (if not already done):

# On your Raspberry Pi: sudo apt update && sudo apt upgrade -y sudo apt install nodejs npm -y sudo npm install -g zigbee2mqtt # Edit config file nano /opt/zigbee2mqtt/data/configuration.yaml # Set serial port to /dev/ttyUSB0 (check with: ls /dev/ttyUSB*) # Set homeassistant: false, permit_join: true

Step 2 — Pair your IKEA bulb:

# Start Zigbee2MQTT sudo systemctl start zigbee2mqtt # Factory reset your IKEA bulb: switch on/off 6 times rapidly # It should flash to confirm reset, then appear in Zigbee2MQTT log # Check: sudo journalctl -u zigbee2mqtt -f --output=cat

Step 3 — Install Python libraries:

pip install requests paho-mqtt --break-system-packages

4.4  The Code — weather_light.py

import requests import paho.mqtt.publish as publish import time MQTT_HOST = "localhost" BULB_TOPIC = "zigbee2mqtt/YOUR_BULB_NAME/set" CITY = "London" # change to your city def get_weather(): """Fetch tomorrow's weather code from Open-Meteo.""" url = (f"https://api.open-meteo.com/v1/forecast" f"?latitude=51.5&longitude=-0.1" f"&daily=weathercode&timezone=Europe/London") r = requests.get(url, timeout=10) codes = r.json()["daily"]["weathercode"] return codes[1] # tomorrow (index 0 = today) def weather_to_light(code): """Map weather code to a calm bulb colour.""" # WMO weather codes: 71-77 = snow, 85-86 = snow showers if code in range(71, 78) or code in [85, 86]: return {"color": {"x": 0.14, "y": 0.07}, "brightness": 120} # Blue # 61-67 = rain elif code in range(61, 68): return {"color": {"x": 0.25, "y": 0.27}, "brightness": 100} # Cool white # 0-3 = clear/partly cloudy elif code in range(0, 4): return {"color_temp": 300, "brightness": 200} # Warm white else: return {"color_temp": 370, "brightness": 150} # Neutral def send_to_bulb(payload): import json publish.single(BULB_TOPIC, json.dumps(payload), hostname=MQTT_HOST) print(f"Sent to bulb: {payload}") if __name__ == "__main__": while True: try: code = get_weather() print(f"Tomorrow weather code: {code}") light_setting = weather_to_light(code) send_to_bulb(light_setting) except Exception as e: print(f"Error: {e}") time.sleep(3600) # check every hour

4.5  Run and Test

Save the script as weather_light.py on your Pi

Update BULB_TOPIC with your actual bulb name from Zigbee2MQTT

Update latitude/longitude for your city (use latlong.net to find them)

Run: python weather_light.py

Watch your bulb change colour — it updates every hour

Auto-start on boot: sudo crontab -e → add @reboot python /home/pi/weather_light.py &

Congratulations! You have just built a working piece of calm technology. Your home now knows about tomorrow's weather — and communicates it to you without a single notification.

5. Experiments to Carry Out

Each experiment builds on the previous one. Complete them in order. For each, record your results in a CSV file using the format shown — this data feeds directly into the research.

Experiment 1  —  Weather Signal Light (Starter)  |  Week 4–5

Already covered in Section 4. Once working, run it for 7 continuous days and log the following:

| Date | Forecast Code | Light Colour Shown | Actual Weather Next Day | Correct? |
| --- | --- | --- | --- | --- |
| 2026-MM-DD | 71 (snow) | Blue | Light snow at 9am | Yes |

Research question: How often does the weather API forecast match actual conditions?

Experiment 2  —  Simultaneous Load Logger  |  Week 5–6

Goal: Quantify the energy problem IoT-GPT is solving. Log the real-world power draw when two high-power devices run simultaneously vs. staggered.

Equipment: 2 smart plugs with energy monitoring (TP-Link Kasa EP25)

Device A: Electric kettle or fan heater (~2,000–2,500W)

Device B: Hair dryer or second heater (~1,000–2,000W)

Method:

Run both devices simultaneously for 10 minutes. Record peak wattage every 30 seconds.

Wait 30 minutes. Run Device A for 15 min, then Device B for 15 min (staggered).

Compare peak demand and calculate cost difference using your electricity tariff.

| Mode | Peak Watts | Duration (min) | Energy (kWh) | Estimated Cost |
| --- | --- | --- | --- | --- |
| Simultaneous | — | 10 | — | — |
| Staggered | — | 10 | — | — |

Experiment 3  —  Multi-Sensor MQTT Network  |  Week 5–6

Goal: Build a 2-node sensor network and demonstrate that a central brain can receive all data and make coordinated decisions.

Node 1: Your Raspberry Pi with DHT22 publishing to home/node1/temperature

Node 2: Simulated second room using sample 04_mqtt_iot_simulator.py

Success criteria: Both nodes publishing; central subscriber receives all messages; latency under 500ms.

Experiment 4  —  Off-Peak Scheduler Validation  |  Week 7

Goal: Verify that the smart scheduler actually moves loads to cheaper electricity hours and calculate real savings.

Find your electricity tariff: standard rate (e.g. £0.24/kWh) and off-peak rate (e.g. £0.07/kWh)

Run sample 03_smart_device_scheduler.py for one week on your Pi

Compare: cost without IoT-GPT (naive scheduling) vs with IoT-GPT (optimised)

Extrapolate to annual savings and log your result

| Week | Naive Cost (£) | Optimised Cost (£) | Saving (£) | Saving (%) |
| --- | --- | --- | --- | --- |
| Week 1 | — | — | — | — |

Experiment 5  —  Full IoT-GPT Mini Demo  |  Week 8

Goal: Integrate all components into a working mini-version of IoT-GPT and record a 24-hour automated operation.

System components required:

DHT22 sensor → publishing to MQTT

Weather API → fetched every hour

Smart bulb → receiving commands from brain

Coordinator script (iot_gpt_brain.py) → making decisions

InfluxDB + Grafana → logging and visualising all activity

Record: How many autonomous decisions did the system make in 24 hours? How many were correct?

6. Project Implementation Plan

This is your end-to-end project timeline from buying hardware to submitting a contribution. Each milestone is a concrete deliverable you can point to.

| Milestone | Week | Deliverable | Status |
| --- | --- | --- | --- |
| M0 — Hardware setup | Week 1 | Pi running, SSH working, Python scripts executing | [ ] |
| M1 — First sensor reading | Week 2 | DHT22 logging to CSV, 24h data collected | [ ] |
| M2 — MQTT network live | Week 3 | Mosquitto + paho publishing sensor data | [ ] |
| M3 — Smart bulb paired | Week 4 | IKEA bulb controllable via Python over MQTT | [ ] |
| M4 — Weather light working | Week 5 | weather_light.py running 24/7, logs started | [ ] |
| M5 — Load experiment done | Week 6 | Simultaneous vs staggered power CSV filed | [ ] |
| M6 — InfluxDB + Grafana live | Week 6 | 48h dashboard showing sensor trends | [ ] |
| M7 — Scheduler validated | Week 7 | Off-peak savings calculated, result logged | [ ] |
| M8 — Mini IoT-GPT complete | Week 8 | 24h autonomous operation recorded | [ ] |
| M9 — Research contribution | Week 10 | GitHub pull request with data, code, or report | [ ] |
| M10 — Experiment report | Week 12 | 2-page write-up of findings submitted via GitHub | [ ] |

Tick off each milestone in the table above as you complete it. Share your progress in the GitHub Discussions tab of the repo.

6.1  How to Contribute on GitHub

Fork the repository: github.com/gsleo1976/iot-gpt-research → Fork

Clone your fork: git clone https://github.com/YOUR_USERNAME/iot-gpt-research

Create a branch: git checkout -b student/your-name-experiment-1

Add your experiment data as CSV in a folder: students/your-name/

Open a Pull Request with a short summary of what you found

Good First Contributions: Experiment result CSVs • Bug fixes in sample code • New sample programs • Improvements to the beginner guide • Translation of docs • New device profiles for the DAR

7. Step-by-Step Setup Guide

Follow these steps in order on your first day with the hardware. Each step should take 10–20 minutes.

Step 1  —  Flash Raspberry Pi OS

Download Raspberry Pi Imager from raspberrypi.com/software

Insert microSD card into your laptop

Open Raspberry Pi Imager → Choose OS → Raspberry Pi OS (64-bit, Lite is fine)

Click the gear icon (advanced options) → enable SSH, set hostname, set WiFi, set username/password

Write to card → wait 3–5 minutes → eject card

Insert card into Pi, connect HDMI + keyboard for first boot (or SSH directly if WiFi is set)

Step 2  —  First Boot and Update

# After login: sudo apt update && sudo apt full-upgrade -y sudo raspi-config # expand filesystem, set locale/timezone, enable SSH # Install essentials sudo apt install -y python3-pip git vim mosquitto mosquitto-clients nodejs npm pip3 install requests paho-mqtt adafruit-circuitpython-dht --break-system-packages

Step 3  —  Connect DHT22 Sensor

Wire the DHT22 to your Pi (3 connections):

| DHT22 Pin | Raspberry Pi Pin | Colour (typical) |
| --- | --- | --- |
| VCC (+) | Pin 1 (3.3V) | Red |
| DATA | Pin 7 (GPIO 4) | Yellow |
| GND (-) | Pin 6 (GND) | Black |

Note: Place a 10kΩ pull-up resistor between VCC and DATA on the breadboard. Without it, readings may be unreliable.

# Test DHT22 reading python3 -c " import adafruit_dht, board, time sensor = adafruit_dht.DHT22(board.D4) time.sleep(2) print(f'Temperature: {sensor.temperature:.1f}C') print(f'Humidity: {sensor.humidity:.1f}%') "

Step 4  —  Configure Mosquitto MQTT Broker

# Create config file sudo nano /etc/mosquitto/conf.d/local.conf # Add these lines: listener 1883 allow_anonymous true # Restart broker sudo systemctl restart mosquitto sudo systemctl enable mosquitto # Test (open two terminals): # Terminal 1: mosquitto_sub -t test/# # Terminal 2: mosquitto_pub -t test/hello -m "world"

Step 5  —  Install and Configure Zigbee2MQTT

# Find your ZigBee dongle port ls /dev/ttyUSB* # should show /dev/ttyUSB0 # Install Zigbee2MQTT sudo npm install -g zigbee2mqtt --unsafe-perm # Create config directory and file sudo mkdir -p /opt/zigbee2mqtt/data sudo nano /opt/zigbee2mqtt/data/configuration.yaml

Paste this into configuration.yaml:

homeassistant: false permit_join: true mqtt: base_topic: zigbee2mqtt server: mqtt://localhost serial: port: /dev/ttyUSB0 frontend: port: 8080

# Start Zigbee2MQTT sudo npx zigbee2mqtt # You should see: "Started Zigbee2MQTT" in the log

Step 6  —  Pair Your IKEA Smart Bulb

Make sure Zigbee2MQTT is running (permit_join: true in config)

Put the bulb in the lamp socket and turn it on

Factory reset the bulb: rapidly switch power ON-OFF-ON-OFF-ON-OFF (6 times) — the bulb will blink to confirm reset

Watch the Zigbee2MQTT log — you should see "New device joined" with a device address

Note the device's friendly name (or set a name in config)

Test control from command line:

# Turn bulb on mosquitto_pub -t "zigbee2mqtt/YOUR_BULB/set" -m '{"state":"ON","brightness":200}' # Set warm white mosquitto_pub -t "zigbee2mqtt/YOUR_BULB/set" -m '{"color_temp":370}' # Set blue (for snow alerts) mosquitto_pub -t "zigbee2mqtt/YOUR_BULB/set" -m '{"color":{"x":0.14,"y":0.07}}' # Turn off mosquitto_pub -t "zigbee2mqtt/YOUR_BULB/set" -m '{"state":"OFF"}'

Step 7  —  Clone the Research Repository and Run Sample Code

# Clone the repo git clone https://github.com/gsleo1976/iot-gpt-research cd iot-gpt-research/samples # Run the weather forecast program python3 01_weather_forecast.py # Run the calm light simulator python3 07_calm_light_simulator.py # Run the MQTT IoT simulator python3 04_mqtt_iot_simulator.py

Step 8  —  Set Up Auto-Start on Boot

# Add to crontab so everything starts automatically after reboot sudo crontab -e # Add these lines: @reboot sleep 10 && sudo systemctl start mosquitto @reboot sleep 15 && cd /home/pi && python3 weather_light.py & @reboot sleep 20 && sudo npx zigbee2mqtt &

You are ready! If all 8 steps completed successfully, you have a working IoT-GPT prototype. Proceed to Section 4 to build the weather-light project, then work through the experiments in Section 5.

8. Troubleshooting Reference

| Problem | Likely Cause | Fix |
| --- | --- | --- |
| DHT22 returns None or RuntimeError | Bad wiring or missing pull-up resistor | Check VCC=3.3V, add 10kΩ resistor between VCC and DATA |
| Can't SSH into Pi | SSH not enabled or wrong IP | Use raspi-config to enable SSH; find IP with nmap or router admin page |
| Mosquitto connection refused | Broker not running | sudo systemctl start mosquitto; check sudo systemctl status mosquitto |
| ZigBee dongle not found at /dev/ttyUSB0 | Wrong port or missing drivers | ls /dev/ttyUSB*; try /dev/ttyACM0; check dmesg ¦ grep usb |
| Bulb not pairing | Not in factory reset mode | Switch on/off 6× rapidly; light blinks = reset successful; retry pairing |
| Weather API returning error | No internet or wrong coordinates | Check ping 8.8.8.8; verify lat/lon values in your script |
| InfluxDB won't start | Port 8086 already in use | sudo lsof -i :8086; kill conflicting process |
| Grafana dashboard shows no data | Wrong InfluxDB connection or bucket name | Verify bucket name; test query in InfluxDB Data Explorer first |
| pip install fails | Missing system packages | sudo apt install python3-dev libgpiod2; retry pip install |

9. Useful Links & Resources

| Resource | URL | What It Is |
| --- | --- | --- |
| Research Repository | github.com/gsleo1976/iot-gpt-research | All code, docs, and sample programs |
| Raspberry Pi Imager | raspberrypi.com/software | Flash OS to microSD card |
| Raspberry Pi GPIO Pinout | pinout.xyz | Interactive Pi pin diagram |
| Zigbee2MQTT Getting Started | zigbee2mqtt.io/guide/getting-started | Pair and control ZigBee devices |
| Open-Meteo API | open-meteo.com | Free weather + AQI API (no key needed) |
| Paho MQTT Python Client | pypi.org/project/paho-mqtt | MQTT library for Python |
| InfluxDB OSS Install | docs.influxdata.com/influxdb/v2/install | Time-series database for sensor data |
| Grafana Installation | grafana.com/docs/grafana/latest/setup-grafana | Dashboard and alerting platform |
| MicroPython (optional) | micropython.org | Python for microcontrollers (ESP32, Pico) |
| MQTT Explorer (desktop app) | mqtt-explorer.com | GUI tool to browse all MQTT topics |

IoT-GPT Research Project  |  Gurdev Singh  |  gurdev.leo@gmail.com  |  github.com/gsleo1976/iot-gpt-research