IoT-GPT Research Project

Beginner's Guide to Experimental Setup

How to Build a Smart Home IoT Research Lab from Scratch

For students and researchers with little to no IoT experience

Version 1.0  |  March 2026

# 1. Introduction

This guide walks you through everything you need to set up the IoT-GPT experimental environment — even if you have never worked with IoT devices, Raspberry Pi, or smart home technology before. By the end of this guide, you will have a working smart home testbed that can run all six experiments described in the IoT-GPT research project.

The guide is organized into clear, numbered steps. Each step includes:

What you need (hardware, software, or knowledge)

Step-by-step instructions

How to verify it worked

What to do if something goes wrong

💡 Tip:  No prior experience with IoT or hardware is required. Each step builds on the previous one. Take your time — the setup typically takes 1-2 days on a first attempt.

## 1.1 What Is IoT-GPT?

IoT-GPT is an intelligent home automation system that:

Learns the attributes of every connected device (power draw, warm-up time, conflicts with other devices, etc.)

Coordinates multiple devices together — for example, scheduling the water heater and room heater so they never run at the same time, avoiding circuit overload

Uses calm technology — ambient light signals instead of phone notifications — to inform you without interrupting your focus

Runs entirely on a Raspberry Pi at the edge of your home network, with no cloud service required

## 1.2 What You Will Build

Your experimental setup will consist of:

1 Raspberry Pi 5 running the full IoT-GPT software stack

2–3 ZigBee-connected smart devices (heaters, smart bulb, or sensors)

A temperature and humidity sensor (DHT22) wired to the Pi GPIO pins

An MQTT network for all device communication

A data logging pipeline using InfluxDB and Grafana dashboards

# 2. What You Need (Shopping List)

## 2.1 Hardware

| Item | Model / Spec | Cost (CAD) | Where to Buy |
| --- | --- | --- | --- |
| Raspberry Pi 5 | 4GB or 8GB RAM | ~$60 | raspberrypi.com / Canada Computers |
| MicroSD Card | 32GB Class 10 minimum | ~$8 | Amazon / Best Buy |
| SD Card Reader | USB adapter (if needed) | ~$5 | Amazon |
| Power Supply | Official Pi 5 PSU (27W USB-C) | ~$15 | raspberrypi.com |
| ZigBee USB Dongle | SONOFF Zigbee 3.0 USB Dongle Plus | ~$20 | Amazon |
| Smart LED Bulb | IKEA TRÅDFRI E26 or Philips Hue | ~$15 | IKEA / Best Buy |
| DHT22 Sensor | Temperature + Humidity | ~$5 | Amazon |
| Jumper Wires | Female-to-Male, pack of 20 | ~$4 | Amazon |
| Breadboard | 400-tie mini breadboard | ~$4 | Amazon |
| Network Cable | CAT5e Ethernet 2m | ~$5 | Any hardware store |
| TOTAL |  | ~$141 CAD | Budget ~$150 for extras |

⚠  Note:  The Raspberry Pi 5 may be out of stock. The Raspberry Pi 4 (4GB) works fine as an alternative.

## 2.2 Software (All Free)

| Software | Purpose | How to Install |
| --- | --- | --- |
| Raspberry Pi OS 64-bit | Operating system | Raspberry Pi Imager tool |
| Python 3.11+ | Running IoT-GPT code | Pre-installed on Pi OS |
| Mosquitto | MQTT broker for device messaging | sudo apt install mosquitto |
| Zigbee2MQTT | Connect ZigBee devices to MQTT | npm install -g zigbee2mqtt |
| InfluxDB 2.x | Store time-series sensor data | Official InfluxDB package |
| Grafana | Visualize sensor data in dashboards | Official Grafana package |
| Node-RED (optional) | Visual IoT programming | sudo npm install -g node-red |
| VS Code + Remote SSH | Code from your laptop | code.visualstudio.com |

# 3. Setting Up the Raspberry Pi

## 3.1 Install the Operating System

Download and install Raspberry Pi Imager on your laptop:

https://www.raspberrypi.com/software/

Open Raspberry Pi Imager and follow these steps:

Click 'CHOOSE DEVICE' and select 'Raspberry Pi 5'

Click 'CHOOSE OS' and select 'Raspberry Pi OS (64-bit)'

Click 'CHOOSE STORAGE' and select your microSD card

Click the gear icon to pre-configure hostname, username, password, and Wi-Fi

Click 'WRITE' and wait for it to finish (~5 minutes)

💡 Tip:  Pre-configuring Wi-Fi and SSH means you can use the Pi without a monitor. Just power it on and connect via SSH from your laptop.

## 3.2 First Boot and SSH Connection

Insert the microSD into the Pi, connect power, and wait 2 minutes. Then SSH in from your laptop:

ssh pi@raspberrypi.local

Update system packages (takes 10–20 minutes):

sudo apt update && sudo apt upgrade -y

⚠  Note:  Do not turn off the Pi while updating. Let the upgrade complete fully.

# 4. Installing the MQTT Broker (Mosquitto)

MQTT is the messaging protocol that IoT devices use to communicate. Think of it like a group chat: devices publish messages to "topics" and other devices subscribe to receive them.

## 4.1 Install Mosquitto

sudo apt install -y mosquitto mosquitto-clients

sudo systemctl enable mosquitto

sudo systemctl start mosquitto

## 4.2 Test the Broker

Open two terminal windows on the Pi. In window 1, subscribe to a test topic:

mosquitto_sub -t 'iot/test' -v

In window 2, publish a message:

mosquitto_pub -t 'iot/test' -m 'Hello from IoT-GPT!'

The message should appear in window 1. If it does, MQTT is working!

## 4.3 Configure for Your Network

Allow connections from devices on your local network:

sudo nano /etc/mosquitto/mosquitto.conf

Add at the end of the file:

listener 1883

allow_anonymous true

Save and restart:

sudo systemctl restart mosquitto

# 5. Connecting Your First Sensor (DHT22)

The DHT22 sensor measures temperature and humidity. It connects directly to the Raspberry Pi GPIO pins using just three wires. No soldering required!

## 5.1 Wiring

| DHT22 Pin | Raspberry Pi Pin | Wire Color |
| --- | --- | --- |
| VCC (+) | Pin 1 (3.3V Power) | Red |
| DATA | Pin 7 (GPIO 4) | Yellow |
| GND (-) | Pin 6 (Ground) | Black |

💡 Tip:  The DHT22 has 4 pins but you only use 3. Leave the third pin (labeled NC = No Connect) empty.

## 5.2 Install the Library

pip3 install adafruit-circuitpython-dht

sudo apt install libgpiod2

## 5.3 Read Sensor Data

Create and run a test script:

import adafruit_dht, board, time

sensor = adafruit_dht.DHT22(board.D4)

for i in range(5):

print(f'Temp: {sensor.temperature}C  Humidity: {sensor.humidity}%')

time.sleep(2)

⚠  Note:  DHT22 sometimes fails on the first read. This is normal. The script retries automatically. If it consistently fails, re-check the wiring.

# 6. Data Logging with InfluxDB and Grafana

InfluxDB stores your sensor data over time like a diary for numbers. Grafana turns that data into real-time charts and dashboards.

## 6.1 Install InfluxDB

sudo apt update && sudo apt install influxdb2

sudo systemctl enable influxdb && sudo systemctl start influxdb

Open setup UI at: http://raspberrypi.local:8086

Create an organization, a bucket named "iot_gpt", and save the API token.

## 6.2 Install Grafana

sudo apt install -y grafana

sudo systemctl enable grafana-server && sudo systemctl start grafana-server

Open Grafana at: http://raspberrypi.local:3000  (default: admin/admin)

💡 Tip:  In Grafana, add InfluxDB as a data source and create a dashboard with temperature and humidity panels for a live view of your sensor.

# 7. Running Your First Experiment

## 7.1 Energy Baseline (Experiment 1 Preview)

Before testing IoT-GPT, measure baseline energy usage for one week with a manual schedule:

Connect a smart power meter (TP-Link Kasa) to your heater outlet

Install: pip3 install python-kasa

Run the baseline energy logger from the samples folder:

python3 08_energy_usage_tracker.py

## 7.2 What Good Data Looks Like

After a week of logging, your Grafana dashboard should show:

Temperature readings arriving every 60 seconds with no gaps

Daily energy usage per device clearly visible in kWh

Peak usage during morning (6–9 AM) and evening (5–8 PM) hours

## 7.3 Auto-Start on Boot

Set your IoT-GPT scripts to start automatically when the Pi boots:

sudo nano /etc/systemd/system/iotgpt.service

[Unit]

Description=IoT-GPT Service

After=network.target

[Service]

ExecStart=/usr/bin/python3 /home/pi/iot-gpt/main.py

Restart=always

User=pi

[Install]

WantedBy=multi-user.target

sudo systemctl enable iotgpt && sudo systemctl start iotgpt

# 8. Troubleshooting Common Problems

| Problem | Likely Cause | Solution |
| --- | --- | --- |
| Can't SSH into Pi | Pi not on network or wrong hostname | Use the Pi's IP address directly. Check your router's device list. |
| DHT22 shows errors | Loose wiring or reading too fast | Check wiring. Add time.sleep(2) between reads. |
| MQTT messages not received | Broker not started or firewall | Run: sudo systemctl status mosquitto |
| InfluxDB will not start | Port conflict or low memory | Check: journalctl -u influxdb. Try rebooting. |
| Grafana shows no data | Wrong data source settings | Re-enter InfluxDB URL and token in Grafana. |
| ZigBee not pairing | Wrong USB port or firmware | Unplug and replug dongle. Check Zigbee2MQTT logs. |
| Python import errors | Library not installed | Run: pip3 install <library_name> |

# 9. Next Steps and Resources

## 9.1 After Basic Setup

Run all 10 sample Python programs in the 'samples/' folder to learn each concept

Connect your ZigBee smart bulb and test calm light notifications (Program 07)

Try the full smart device scheduling simulation (Program 03)

Start the 7-day energy baseline measurement for Experiment 1

Join the weekly research team meetings (schedule on GitHub)

## 9.2 Useful Resources

| Resource | URL / Location | What It Is |
| --- | --- | --- |
| Project GitHub | github.com/gsleo1976/iot-gpt-research | All code, docs, and data |
| Raspberry Pi Docs | raspberrypi.com/documentation | Official Pi guides |
| MQTT Essentials | hivemq.com/mqtt-essentials | Free MQTT learning series |
| InfluxDB Docs | docs.influxdata.com | Database setup and queries |
| Zigbee2MQTT | zigbee2mqtt.io | Connect any ZigBee device |
| Adafruit DHT Guide | learn.adafruit.com/dht | Sensor wiring and code |

## 9.3 Getting Help

Post questions in the team shared chat (link in GitHub repo)

Weekly lab sessions: every Saturday 10am–12pm (virtual)

Email the project lead: gurdev.leo@gmail.com

GitHub Issues: open an issue at the repository for technical bugs

You're ready to start! The best way to learn IoT is to build something.

Run the sample programs, break things, fix them, and have fun!

— IoT-GPT Research Team